# How to calculate the period of compound pendulum with liquid damping?

WANG JINTIAN, 6597, Guanghua Cambridge International School

# Introduction

Pendulums have long held a prominent role in our everyday lives, particularly within the fields of physics. These devices can be seen in various forms, from the playful swings that bring joy to children to the implementation of tuned mass dampers, which enhance the stability and safety of towering skyscrapers (Heffernan, 2015), and even extends to the field of rocketry, where a dual shock absorber system is employed to ensure the smooth launch by reducing excessive vibrations (Malik, 2008).

In physics education, the simple pendulum is a commonly taught concept (Suzuki, 2008), but scientists have expanded their research to explore the intricacies of compound pendulum systems (Hinrichse, 1981). However, most studies in this area have focused solely on ideal situations without considering the effects of friction (Feynman, 1963). It is important to correct these idealized models and account for damping conditions which can cause non-linear motion combined with chaos (Ji, Zhou, & Li, 2018), and difficult to calculate and understand (Peters, 2003). By incorporating damping effects, scientists can develop more accurate models and gain a better understanding of the real-world behavior of compound pendulums.

While some researchers have directed their attention towards investigating the effects of air friction on compound pendulums (Zhang, Wang, Qu, & Zou, 2008), an emerging area of interest lies in exploring the influence of liquid damping. By studying the behavior of these systems, we can gain valuable insights into their dynamics and further our understanding of their real-world applications.

When it comes to liquid damping situations, the theoretical calculation of the magnitude of viscous resistance is quite complex, which includes viscosity, mass, length, cross-sectional area and so on. However, when discussing the effect of fluids damping, two simplified models are commonly used, which one assumes that the friction acting on the pendulum is proportional to the velocity, while the other assumes that the friction is proportional to the square of the velocity (Lü, 2006). In this essay, we will calculate the period of compound pendulums with liquid damping using two previous approximation methods and also compare the differences between these two methods and experimental results.

Fig 1. A free body diagram of a compound pendulum

θ

f

F

R

W

ω

O

C

Derivation

## 2.1 Liquid friction proportional to velocity

As we can see from Fig.1, assume the pendulum is acted by a gravitational force W, which is fixed at point O and has the center of mass at C. The distance between axes O and C is R, and the angle between the vertical line and OC is θ, while the pendulum is swing at angular velocity ω. Examining the forces acting on the pendulum reveals three components: gravitational force W, buoyance F, accounting for complete liquid submersion, the third is the most important, and the crucial frictional force f. In this situation, it is proportional to velocity, i.e.

f=-kv=-kRω

k here stands for the damping constant of the pendulum, and f is minus since it is opposite to the direction of rotation. According to analogue Newton Second Law in rotation

Iα=M	(1)

R.H.S. is the sum of all the moments, which can be expanded. By substituting all the forces and their moments, equation (1) can be written as

Iθ=WR+fR+FR

Expand each term.

Iθ=-mgRsinθ-kωR2+ρgVRsinθ	(2)

Notice that weight is also opposite to the direction of rotation, so there should be another minus sign before it. Then, (2) can be rewritten in differential form.

Iθ+kR2θ+m-ρVgRsinθ=0	(3)

For small θ, which θ≪5°, we have sinθ≈θ. There is discussion of whether small angle is possible or not, experiments later will demonstrate this problem and proof that it is available in liquid damping.

i.e. (3) can be written in

θ+kR2Iθ+m-ρVgRIθ=0

Let r1=kR2I , r2=m-ρVgRI , and

d2θdt2+r1dθdt+r2θ=0	(4)

This is a homogeneous second-order linear ordinary differential equation. Notice that ddxefx=ddxfxefx, which the nth derivative of efx is ddxfxnefx, original form times a coefficient, we can promote into (4). Assume θ=Aemt, so θ=Am emt= m, θ=m2. Substitute into (4)

m2θ+r1mθ+r2θ=0

Each term of the equation has θ, by simplifying it there is

m2+r1m+r2=0

This is a linear equation in two unknowns. By find m we can solve θ, where ∆=b2-4ac=r12-4r2, where ∆=i4r2-r12, i=-1. According to quadratic formula x=-b±b2-4ac2a,

m=-r1±i4r2-r122I

According to Cambridge International AS & A Level Further Mathematics Coursebook, pp. 550-556, if a homogeneous second-order linear ordinary differential equation has roots of auxiliary equation λ1=a+bi, λ2=a-bi, then the complementary form is given by y=eaxA cos bx+B sin bx. i.e.

θ=e-r12tAcosi4r2-r122t+B sini4r2-r122t	(5)

This is the solution to (4). The presence of complex numbers in the solution represents the system's behavior in phase space. To get the actual period of the pendulum, we should extract the real part of the function. From DeMoivre’s theorem, cosnθ+isinnθ=cosθ+isinθn=einθ,

cosi4Ir2-r122t=ei24r2-r12t    sini4Ir2-r122t=e-i24r2-r12t

i.e.

θ=e-r12tAei24r2-r12t+B e-i24r2-r12t

=e-r12tA+B cos4r2-r122t+iA-B sin4r2-r122t	(6)

The real part of (6) is

θ=Ce-r12tcos4r2-r122t	(7)

This is a cosine function of t, where Ce-r12t is the coefficient and decrease with the time goes. This suits the real action in damping condition, that as time goes by, pendulum swings with a smaller and smaller angle, but time taken for each cycle doesn’t change.

To calculate the period of a trigonometry function, we should first find out the angular frequency, which is ω. Therefore, since the original period of cosx is 2π, the current period can be calculated by T=2πω.

Within (7), ω=4Ir2-r122, thus

T=2πω=2π4r2-r122=4π4r2-r12

Recall that r1=kR2I , r2=m-ρVgRI,

T=4π4m-ρVgRI-k2R4I2

(8)

And this is the formula of the condition where damping force is directly proportional to the velocity.

## 2.2 Liquid friction proportional to the square of velocity

In the second situation, the friction acting on the compound pendulum is proportional to the square of velocity, that is

f=-kv2=-kR2ω2

Again, k here stands for the damping constant of the pendulum. Hence (2) can be rewritten in

Iθ=-mgRsinθ-kω2R3+ρgVRsinθ	(9)

Notice that the second term has been change into -kω2R3, which refers to the moment of the friction acting on the pendulum, where M=fR=-kω2R2∙R=-kω2R3. This term is also negative because it is in the opposite direction of rotation. Rearrange it into differential form we have

Iθ+kR3θ2+m-ρVgRsinθ=0	(10)

Also θ is a small angle that θ≪5°, as a result we have sinθ≈θ.

θ+kR3Iθ2+m-ρVgRIθ=0

Ler1=kR3I, r2=m-ρVgRI , afterwards

d2θdt2+r1dθdt2+r2θ=0	(11)

This represents a second-order non-linear ordinary differential equation, distinct from our previous solution. A new mathematical approach is required due to the presence of a square on the first derivative. Assume  y=dθdt2, where is a function about θ. According to

ddθy=ddθdθdt2=2d2θdt2

(11) can be shifted into another differential equation about y.

dydθ+2r1y+2r2θ=0	(12)

Here (12) is a linear differential equation. By solving y we can get the function of θ through some extent. However, details still need to be listed due to this is not an easy job. Convert (12) into

dy+2r1y+r2θdθ=0	(13)

Let m=2r1y+r2θ, where dm=2r1dy+2r2dθ, and (13) can be transferred into dy+mdθ=0. According to m , y=m2r1-r2r1θ. Differentiate y, dy=dm2r1-r2dθr1. Use instead into (13).

dm2r1-r2dθr1+mdθ=0

Simplify the equation and we get an integrable expression of dm and dθ.

dm2r2-2r1m=dθ

Upon integration, the function m is determined as:

m=Ae2r1θ-r2r1

Take into y=m2r1-r2r1θ,
y=Ae2r1θ-r2r1θ+r22r12

Where A is integral constant. To calculate A, consider when θ=θ0, i.e. maximum amplitude, dθdtθ=θ0=0, along with y=dθdt2=0. Through substitution,

A=r22r122r1θ0-1e2r1θ0	(14)

Now y and A have been worked out, and it should be followed by deriving θ from y, which

Ae2r1θ-r2r1θ+r22r12=dθdt2

Take the square root of both LHS and RHS sides. Separate the variables.

Ae2r1θ-r2r1θ+r22r12=dθdt

Expand and integrate both sides.

dt=Ae2r1θ-r2r1θ+r22r12-12dθ

(15)

From using Maclaurin series ex=n=0∞xnn!, the approximate value of e can be simply calculated by the first three terms. And

e-2r1θ=1+-2r1θ1!+-2r1θ22!	(16)

Take into (15), RHS can be exchanged into

A+r22r12-2Ar1+r2r1θ+2Ar12θ2-12dθ

(17)

By completing the square method, notice that

A+r22r12-2Ar1+r2r1θ+2Ar12θ2=A2-r224r14+2A12+r24r12A-r1θ2

And (17) is equivalent to

-8Ar224r14-A21-2Ar1θ-A+r22r122r224r14-A2dθ

(18)

From ∫11-x2ⅆx=sin-1x, take (18) into (15).

∆t=--2Ar1sin-12Ar1θ-A+r22r12r224r14-A2

(19)

Arrange (19), express θ in t.

θ=A+r22r122Ar1-r224r14A2-12r1sinr1-A2t

(20)

In (20), the angular frequency ω=r1-A2. Recall A=r22r122r1θ0-1e2r1θ0.

ω=r1-A2=r1-2r1r2θ0-r2e2r1θ0 4r12=12r2-2r1r2θ0e-2r1θ0

Refer to (16)

ω=12r2-2r1r2θ0e-2r1θ0=12r2-2r1r2θ01-2r1θ0+2r12θ02

Recall r1=kR3I, r2=m-ρVgRI,

ω=12m-ρVgRI-2m-ρVgkR4I2θ01-2kR3Iθ0+2k2R6I2θ02=12m-ρVgRI-2kR3θ0I2-2kR3Iθ0+2k2R6θ02

and from T=2πω, the period of oscillation can be calculated directly.

T=2πω=2π12m-ρVgRI-2kR3θ0I2-2kR3Iθ0+2k2R6θ02=4πm-ρVgRI-2kR3θ0I2-2kR3Iθ0+2k2R6θ02

(21)

## 2.3 Checking and Verifications

In the culmination of our preceding computations, both methodologies have adeptly provided theoretical insights into the period of a compound pendulum influenced by liquid damping under distinct conditions. It is noteworthy that the utilization of approximations in both approaches serves the purpose of streamlining the differential equation into a second-order form. The judicious expansion of the exponential function, specifically aimed at obtaining an integrable function featuring a periodic component, such as the sin function.

In a general sense, we express the periods T as follows:

1. For the assumption where f=-kv=-kRω:

T=4π4m-ρVgRI-k2R4I2

2. For the assumption where f=-kv2=-kR2ω2:

T=4πm-ρVgRI-2kR3θ0I2-2kR3Iθ0+2k2R6θ02

To assess the accuracy of both formulations, we evaluate the scenarios where k = 0 and ρgV = 0, signifying a scenario where the compound pendulum remains devoid of any liquid submersion and friction. Remarkably, in both cases, the derived periods converge to T=2πImgR, aligning with the theoretical period for ideal non-energy loss oscillations. This consistency reinforces the validity of our methodology.

An additional scrutiny is applied to the small angle approximation. Both approaches assert that for θ≪5°,sinθ≈θ. However, a critical inquiry arises: can small angles be pertinent in the presence of substantial damping, as encountered in mediums like water, oil, or ethanol? Esteemed researchers (Zhen, Jia, & Fang, 2004; Alciatore & Histand, 2007) introduce a physical quantity δ, signifying the damping ratio where δ = actual dampingcritical damping. And over damping conditions occur when δ>1, leading to the absence of observable periodic swings. To avert such scenarios and ascertain the suitability of liquid damping for most pendulums, experiments must be conducted to determine the feasibility of assuming small angles. This empirical validation is essential for refining our models and ensuring their accuracy under real-world conditions.

Moreover, while the two mentioned approximation methods provide distinct approaches for determining the period of a compound pendulum, they do not offer guidance on the preferred choice for predicting the pendulum's motion. For a more comprehensive understanding, additional experiments are imperative. These experiments serve to assess and compare the accuracy of both methods, thereby enabling the formulation of well-founded conclusions through a meticulous analysis of the experimental outcomes.

# Experiments

## 3.1 Rough Experiments

The preceding discussion emphasized the importance of experimental verification, prompting the need to assess the validity of assuming small angles. To tackle this, a rough experiment was carefully designed.

In the experiment, a long screw with a textured surface was chosen to simulate a compound pendulum. The textured surface was intentionally picked to highlight the impact of liquid damping on the pendulum's motion. It is noteworthy that a hole was carefully drilled through the screw to accommodate the axes for support. An iron support was strategically positioned to provide a stable horizontal foundation for the entire system. Copper wire served as the rotation axes, and though occasional loosening could appear, it was expected to have a minimal effect on the pendulum's period.

- pictures from the experimental site

- schematic diagram

Fig 2. Rough Experiment

| No. | Equipment Name | Device Model | Role |
| --- | --- | --- | --- |
| 1 | Water tank | 12cm * 12cm *25cm | Contain water |
| 2 | Iron stand | 30cm in length | Provide horizontal foundation |
| 3 | Clamps | Iron with rubber | Hold the entire system |
| 4 | Wooden block | 15cm * 5cm * 1cm | Support the axes |
| 5 | Copper wire | 7mm in diameter | Rotational axes |
| 6 | Aluminum screw | M8, 121.5mm in length | Compound pendulum |
| 7 | Camera | iPhone 11 *0.125 in slow-motion | Record the experiment |

Table 1. Equipment list of rough experiments

To enhance measurement precision, the incorporation of cameras and slow-motion technology was employed. Video editing tools were subsequently utilized to tally the frames encompassing a complete oscillation period. Variable control was not implemented since this is an initial stage of validation, resulted that only a singular set of data was amassed. The experimental procedure was repeated for four times, with the computation of an average value.

| No. | Frame/f | 10T/s | T/s |
| --- | --- | --- | --- |
| 1 | 740 | 6.167 | 0.614 |
| 2 | 736 | 6.133 |  |
| 3 | 732 | 6.100 |  |
| 4 | 738.5 | 6.154 |  |

| Variable | Value | Unit |
| --- | --- | --- |
| k | 0.84±0.02 | kg s-1 |
| I | 0.060±0.003 | g m2 |
| m | 36.0±0.5 | g |
| V | 5.00±0.05 | ml |
| R | 25.7±0.1 | mm |

Table 2. Data of rough experiments

The obtained results were organized in a data table to determine the average outcome. However, special consideration was given to the performance during extended continuous swings. Over 60 periods were observed until the pendulum came to a halt, taking more than 40 seconds to complete. This observation supported the notion that small angles could be reliably used as an approximation under conditions of liquid damping.

While the primary goal of this rudimentary experiment was to confirm assumptions regarding small angle considerations, there is potential for additional calculations to be conducted. Taking all the variables, throughout the first method can get T1=0.60±0.04 s. Simultaneously, the outcome of the second method is T2=0.58±0.06 s. Notably, the results exhibit only a ±5% deviation from the actual value, affirming the accuracy of both formulas concurrently.

## 3.2 Formal Experiments

In the formal experiments, a cube was designed to act as a box of containing masses with various weight and density, in order to change the inertia of the compound pendulum by varying the position of

# References

Alciatore, D. G., & Histand, M. B. (2007). Introduction to Mechatronics and Measurement (3rd ed.).

Feynman, R. P. (1963). The Feynman Lectures on Physics (Volume I). California.

Heffernan, T. (2015, March 18). The World's Second-Tallest Building Sways, But Here's Why You Can't Feel It. Retrieved December 3, 2023, from Popular Mechanics: https://www.popularmechanics.com/technology/infrastructure/a14564/the-121-story-tower-that-never-sways/

Hinrichse, P. F. (1981, May). Practical Applications of the Compound Pendulum. The Physics Teacher, pp. 286-292. Retrieved December 3, 2023

Ji, P., Zhou, L., & Li, L. (2018). Chaotic Motions for a Class of the Compound Pendulum System. International Conference on Applied Mathematics, Modeling and Simulation (AMMS). Hangzhou.

Lü, J. (2006). University Physics Concise Tutorial. Beijing: Tsinghua University Press.

Malik, T. (2008, August 20). Shock Absorber Plan Set for NASA's New Rocket. Retrieved Decemeber 3, 2023, from Space: https://www.space.com/5746-shock-absorber-plan-set-nasa-rocket.html

Mckelvey, L., & Crozier, M. (2018). Cambridge International AS & A Level Further Mathematics Coursebook. Cambridge: Cambridge University Press.

Peters, R. D. (2003). Nonlinear Damping of the 'Linear' Pendulum. arXiv: Classical Physics. Retrieved 12 3, 2023, from https://arxiv.org/abs/physics/0306081

Suzuki, M. &. (2008). Physics of simple pendulum.

Zhang, T., Wang, Y., Qu, G., & Zou, D. (2008, November). Influence of air resistance on the vibration period of compound pendulum. Physics Experimentation, 28(11), 42-45. Retrieved December 4, 2023

Zhen, Y., Jia, Q., & Fang, X. (2004). Mechanics. Beijing: Higher Education Press.